Bluetooth-Controlled Home Automation System

Objective:  Design a system to switch home appliances ON/OFF using Bluetooth via a smartphone.

1. Components Required

- Microcontroller: Arduino UNO REV3

- Bluetooth Module: HC-05 (or HC-06)

- Relay Module: 4-Channel Relay (5V)

- Power Supply: 5V Adapter (for Arduino & Relays)

- Breadboard & Jumper Wires

- Smartphone App: "Bluetooth Terminal" (e.g., Arduino Bluetooth Controller)

- Devices to Control: Bulbs, Fans, etc. (AC/DC)

2. Circuit Diagram

3. Connections:

| Arduino | HC-05 Bluetooth | Relay Module |
| --- | --- | --- |
| 5V | VCC | VCC |
| GND | GND | GND |
| TX (Pin 1) | RX | - |
| RX (Pin 0) | TX | - |
| Digital Pins (2,3,4,5) | - | IN1, IN2, IN3, IN4 |

Note:

- For AC Appliances: Connect relay COM to live wire, NO (Normally Open) to the device.

- For DC Appliances: Connect relay COM to power (+), NO to the device.

4. Working Demo Steps

Step 1: Hardware Setup

- Connect the HC-05 Bluetooth module to Arduino (5V, GND, TX→RX, RX→TX).

- Connect relay inputs (IN1-IN4) to Arduino digital pins (2-5).

- Power the Arduino and relay module.

Step 2: Upload Code

- Upload the provided Arduino code via USB.

- Open Serial Monitor (9600 baud) to check Bluetooth connection.

Step 3: Pair Bluetooth & Control Devices

- Pair HC-05 with your smartphone (Default PIN: 1234 or 0000).

- Open Bluetooth Terminal App (e.g., "Arduino Bluetooth Controller").

- Send commands:

  - A → Relay 1 ON

  - a → Relay 1 OFF

  - B → Relay 2 ON

  - b → Relay 2 OFF

  - (Similarly for relays 3 & 4)

Expected Output:

- When you send A, the corresponding relay activates, turning ON the connected device.

- Sending a turns it OFF.

5. Enhancements (Optional)

- Voice Control: Use an app like "Voice Bluetooth Terminal" for voice commands.

- WiFi + Bluetooth: Replace Arduino with ESP32 for dual control (Bluetooth + WiFi).

- Automation: Add sensors (PIR, LDR) for automatic switching.

Conclusion

This system provides a low-cost, wireless way to control home appliances via Bluetooth. The circuit is simple, and the code can be extended for more devices.